| Students Name | SAP ID |
| --- | --- |
| Sahar Ayub | 70172989 |

CN Project

For

Software Engineering

| Instructor | Sir. Habib ullah Shah |
| --- | --- |
| Semester | Fall 2025 |

Department of Software Engineering

The University of Lahore, Lahore, Pakistan

VIC MODERN HOTEL

- (COMPUTER NETWORKS PROJECT DOCUMENTATION)

- 1. Introduction

- This project presents the design and implementation of a structured computer network for Vic Modern Hotel, developed as part of the Computer Networks course. The goal of the project is to design a realistic, scalable, and secure enterprise network using Cisco Packet Tracer, implementing core networking concepts such as VLANs, subnetting, routing, DNS, DHCP (conceptually), and OSPF.

- The hotel consists of three floors, each hosting multiple departments with specific networking needs. The designed network ensures full inter-department communication, logical separation of traffic, and efficient routing, while following real-world enterprise networking practices.

- 2. Organizational Requirements

- Vic Modern Hotel has the following structure:

- 1st Floor

- Reception

- Store

- Logistics

- 2nd Floor

- Finance

- Human Resources (HR)

- Sales / Marketing

- 3rd Floor

- Information Technology (IT)

- Administration (Admin)

- Each department requires:

- Dedicated network segmentation

- Wired and wireless connectivity

- A network printer

- Communication with all other departments

- 2.1 Screenshot (Flooring):

- 3. Network Design Overview

- 3.1 Physical Topology

- Three routers, one per floor, all placed in the IT Server Room

- Routers interconnected using serial DCE cables

- One switch per floor

- End devices include:

- PCs

- Printers

- Servers

- Wireless devices (laptops and phones via Wi-Fi)

- 3.2 Logical Topology

- Each department is placed in a separate VLAN

- Inter-VLAN routing is handled by routers

- OSPF (Open Shortest Path First) is used for dynamic routing

- All IP addresses are manually assigned

- 3.3 Screenshot (Topology):

- VLAN and IP Addressing Scheme

  - Class-based IP Allocation

- The Vic Modern Hotel network uses a hierarchical IP addressing scheme based on IP classes per floor, followed by VLSM (Variable Length Subnet Masking) to efficiently allocate addresses to individual departments. This design ensures scalability, efficient address utilization, and logical separation of traffic.

| - Floor | - Departments | - IP Class | - Network Block |
| --- | --- | --- | --- |
| - 3rd Floor | - IT, Admin, Servers | - Class A | - 10.0.0.0/16 |
| - 2nd Floor | - Sales, HR, Finance, Server | - Class B | - 172.16.1.0/16 |
| - 1st Floor | - Store, Reception, Logistics | - Class C | - 192.168.1.0/24 |

- Higher floors with critical systems are allocated larger address spaces

- Follows real enterprise hierarchical IP planning

- Simplifies future expansion and troubleshooting

- 4.2 VLAN Allocation

| - Floor | - Department | - VLAN ID | - Network Address |
| --- | --- | --- | --- |
| - 1st | - Reception | - VLAN 70 | - 192.168.1.0/28 |
| - 1st | - Store | - VLAN 60 | - 192.168.1.16/29 |
| - 1st | - Logistics | - VLAN 80 | - 192.168.1.32/28 |
| - 2nd | - Sales | - VLAN 30 | - 172.16.1.0/27 |
| - 2nd | - HR | - VLAN 40 | - 172.16.1.32/28 |
| - 2nd | - Finance | - VLAN 50 | - 172.16.1.48/28 |
| - 3rd | - IT | - VLAN 10 | - 10.0.0.0/26 |
| - 3rd | - Admin | - VLAN 20 | - 10.0.0.64/27 |
| - 3rd | - Core Servers | - VLAN 99 | - 10.0.0.96/28 |

- 4.3 Purpose for VLANs

- Improves security by isolating departments

- Reduces broadcast traffic

- Simplifies management

- Matches real-world enterprise hotel environments

4.4 Screenshot

- VLAN (1st Floor)

- VLAN (2nd Floor)

- VLAN (3rd Floor)

- 5. VLAN-Wise Subnetting Using VLSM

- To avoid IP wastage and meet departmental size requirements, VLSM is implemented within each floor’s reserved network.

- 5.1 3rd Floor – IT, Admin, and Core Servers

- (Hotel Backbone Floor)

- Base Network: 10.0.0.0/16

- Departmental Requirements

| - VLAN | - Department | - Purpose |
| --- | --- | --- |
| - 10 | - IT | - Large staff, testing systems |
| - 20 | - Admin | - Management and control |
| - 99 | - Core Servers | - PMS, billing, database, CCTV |

- Subnet Size Decisions

| - Department | - Required Hosts | - Selected Prefix |
| --- | --- | --- |
| - IT | - ~50+ | - /26 |
| - Admin | - ~20 | - /27 |
| - Servers | - ~10 | - /28 |

- 3rd Floor VLSM Allocation

| - VLAN | - Department | - Network | - Prefix | - Usable IP Range | - Broadcast |
| --- | --- | --- | --- | --- | --- |
| - 10 | - IT | - 10.0.0.0 | - /26 | - 10.0.0.1 – 10.0.0.62 | - 10.0.0.63 |
| - 20 | - Admin | - 10.0.0.64 | - /27 | - 10.0.0.65 – 10.0.0.94 | - 10.0.0.95 |
| - 99 | - Servers | - 10.0.0.96 | - /28 | - 10.0.0.97 – 10.0.0.110 | - 10.0.0.111 |

- 5.2 2nd Floor – Sales, HR, Finance

- (Hotel Operations Floor)

- Base Network: 172.16.1.0/16

- Departmental Requirements

| - VLAN | - Department | - Purpose |
| --- | --- | --- |
| - 30 | - Sales | - Corporate bookings, events |
| - 40 | - HR | - Staff management |
| - 50 | - Finance | - Billing and payroll |
| - 98 | - Dept Server | - Local applications |

- Subnet Size Decisions

| - Department | - Required Hosts | - Prefix |
| --- | --- | --- |
| - Sales | - ~25 | - /27 |
| - HR | - ~10 | - /28 |
| - Finance | - ~10 | - /28 |
| - Server | - ~6 | - /29 |

- 2nd Floor VLSM Allocation

| - VLAN | - Department | - Network | - Prefix | - Usable IP Range | - Broadcast |
| --- | --- | --- | --- | --- | --- |
| - 30 | - Sales | - 172.16.1.0 | - /27 | - 172.16.1.1 – 172.16.1.30 | - 172.16.1.31 |
| - 40 | - HR | - 172.16.1.32 | - /28 | - 172.16.1.33 – 172.16.1.46 | - 172.16.1.47 |
| - 50 | - Finance | - 172.16.1.48 | - /28 | - 172.16.1.49 – 172.16.1.62 | - 172.16.1.63 |
| - 98 | - Server | - 172.16.1.64 | - /29 | - 172.16.1.65 – 172.16.1.70 | - 172.16.1.71 |

- 5.3 1st Floor – Reception, Store, Logistics

- (Guest-Facing and Operational Floor)

- Base Network: 192.168.1.0/24

- Departmental Requirements

| - VLAN | - Department | - Purpose |
| --- | --- | --- |
| - 70 | - Reception | - Front desk operations |
| - 60 | - Store | - Inventory management |
| - 80 | - Logistics | - Supplies and movement |

- Subnet Size Decisions

| - Department | - Required Hosts | - Prefix |
| --- | --- | --- |
| - Reception | - ~12 | - /28 |
| - Store | - ~8 | - /29 |
| - Logistics | - ~15 | - /28 |

- 1st Floor VLSM Allocation

| - VLAN | - Department | - Network | - Prefix | - Usable IP Range | - Broadcast |
| --- | --- | --- | --- | --- | --- |
| - 70 | - Reception | - 192.168.1.0 | - /28 | - 192.168.1.1 – 192.168.1.14 | - 192.168.1.15 |
| - 60 | - Store | - 192.168.1.16 | - /29 | - 192.168.1.17 – 192.168.1.22 | - 192.168.1.23 |
| - 80 | - Logistics | - 192.168.1.32 | - /28 | - 192.168.1.33 – 192.168.1.46 | - 192.168.1.47 |

- 5.4 Custom Subnet Mask for each VLAN

- 3rd Floor – IT, Admin & Core Servers

- Base Network: 10.0.0.0/16 (Class A)

| - VLAN | - Department | - Prefix | - Subnet Mask | - Usable Hosts | - Why This Mask |
| --- | --- | --- | --- | --- | --- |
| - 10 | - IT | - /26 | - 255.255.255.192 | - 62 | - IT has most staff, systems, and future growth |
| - 20 | - Admin | - /27 | - 255.255.255.224 | - 30 | - Moderate number of management devices |
| - 99 | - Core Servers | - /28 | - 255.255.255.240 | - 14 | - Few but critical servers |

- 2nd Floor – Sales, HR, Finance

- Base Network: 172.16.1.0/16 (Class B)

| - VLAN | - Department | - Prefix | - Subnet Mask | - Usable Hosts | - Why This Mask |
| --- | --- | --- | --- | --- | --- |
| - 30 | - Sales | - /27 | - 255.255.255.224 | - 30 | - Sales team with growth potential |
| - 40 | - HR | - /28 | - 255.255.255.240 | - 14 | - Small department |
| - 50 | - Finance | - /28 | - 255.255.255.240 | - 14 | - Small, secure department |
| - 98 | - Dept Server | - /29 | - 255.255.255.248 | - 6 | - Limited number of servers |

- 1st Floor – Reception, Store, Logistics

- Base Network: 192.168.1.0/24 (Class C)

| - VLAN | - Department | - Prefix (CIDR) | - Subnet Mask | - Usable Hosts | - Why This Mask |
| --- | --- | --- | --- | --- | --- |
| - 70 | - Reception | - /28 | - 255.255.255.240 | - 14 | - Front-desk PCs and terminals |
| - 60 | - Store | - /29 | - 255.255.255.248 | - 6 | - Very few devices |
| - 80 | - Logistics | - /28 | - 255.255.255.240 | - 14 | - Moderate operational devices |

- 6. Inter-Router Connectivity

- The three routers in the Vic Modern Hotel network are interconnected using serial DCE links to enable communication between all floors, forming the hotel backbone network.

- Inter-Router Networks Used

- 192.168.100.0/30

- 192.168.100.4/30

- 192.168.100.8/30

- Each /30 network connects exactly two router interfaces, ensuring efficient and reliable WAN-style communication between the routers.

- Justification:

- /30 subnets provide exactly two usable IP addresses, ideal for router-to-router communication

- Prevents IP address wastage

- Represents real-world WAN-style enterprise backbone links

- 6.1 IP Addressing Table (Serial Links)

- Link 1

- Router 1 (Se0/2/0) ↔ Router 3 (Se0/2/0)

- Network: 192.168.100.0 /30

| - Router | - Interface | - IP Address | - Subnet Mask |
| --- | --- | --- | --- |
| - R1 | - Se0/2/0 | - 192.168.100.1 | - 255.255.255.252 |
| - R3 | - Se0/2/0 | - 192.168.100.2 | - 255.255.255.252 |

- (Broadcast: 192.168.100.3)

- Link 2

- Router 3 (Se0/2/1) - Router 2 (Se0/2/0)

- Network: 192.168.100.4 /30

| - Router | - Interface | - IP Address | - Subnet Mask |
| --- | --- | --- | --- |
| - R3 | - Se0/2/1 | - 192.168.100.5 | - 255.255.255.252 |
| - R2 | - Se0/2/0 | - 192.168.100.6 | - 255.255.255.252 |

- (Broadcast: 192.168.100.7)

- Link 3

- Router 2 (Se0/2/1) - Router 1 (Se0/2/1)

- Network: 192.168.100.8 /30

| - Router | - Interface | - IP Address | - Subnet Mask |
| --- | --- | --- | --- |
| - R2 | - Se0/2/1 | - 192.168.100.9 | - 255.255.255.252 |
| - R1 | - Se0/2/1 | - 192.168.100.10 | - 255.255.255.252 |

- (Broadcast: 192.168.100.11)

- 5.1 Screen-shots

- (All Serial DCE links Screen-shots not attached as that would increase file size.)

- 7. Router Interface Summary

- 7.1 F1-Router (1st Floor)

- Inter-Router Interfaces

- 192.168.100.1

- 192.168.100.10

- VLAN Networks

- 192.168.1.0/28 (VLAN 70 – Reception)

- 192.168.1.16/29 (VLAN 60 – Store)

- 192.168.1.32/28 (VLAN 80 – Logistics)

- 7.2 F2-Router (2nd Floor)

- Inter-Router Interfaces

- 192.168.100.6

- 192.168.100.9

- VLAN Networks

- 172.16.1.0/27 (VLAN 30 – Sales)

- 172.16.1.32/28 (VLAN 40 – HR)

- 172.16.1.48/28 (VLAN 50 – Finance)

- 172.16.1.64/29 (Department Server)

- 7.3 F3-Router (3rd Floor)

- Inter-Router Interfaces

- 192.168.100.2

- 192.168.100.5

- VLAN Networks

- 10.0.0.0/26 (VLAN 10 – IT)

- 10.0.0.64/27 (VLAN 20 – Admin)

- 10.0.0.96/28 (Core Servers)

- 8. OSPF Routing Configuration

- OSPF is configured on all routers to advertise:

- All VLAN subnets

- All inter-router /30 networks

- Benefits of OSPF:

- Automatic route learning

- Fast convergence

- Scalable enterprise routing

- Supports future hotel expansion

- Screen-shots:

- (The CLI commands run during the working on project were lost later in the .pkt file, and so are not included in the documentation.)

- 9. Printers and End Devices

- Each department has its own network printer

- Printers are assigned:

- Static IP addresses

- Correct VLAN membership

- Justification:

- Reflects real hotel departmental operations

- Supports shared resources within departments

- 10. Security and Network Efficiency

- VLAN separation enhances security

- OSPF reduces routing overhead

- Controlled broadcast domains improve performance

- Logical segmentation supports future scalability

- 11. Testing and Verification

- The following tests were performed in Packet Tracer:

- Ping tests between all VLANs on same floors

- Ping tests between all VLANs on different floors

- Printer access from respective departments

- Inter-floor communication verification

- DNS resolution testing

- Result:

- All devices successfully communicate with each other as required.

- 12. Conclusion

- This project successfully implements a fully functional enterprise-level hotel network that meets all instructor and organizational requirements. The design demonstrates practical application of:

- VLANs and subnetting

- OSPF dynamic routing

- Server-based services (DNS)

- Wired and wireless integration

- The network is realistic, scalable, secure, and operational, and if deployed in a real-world hotel environment, it would function effectively.

- Thank you!